Patient Monitor System

Technology Summary & Firmware Reference

# 1. What this system does

An ESP32-based bedside unit reads a patient's heart rate and blood oxygen concentration (SpO2) from a MAX30100 pulse oximeter, and body temperature from an NTC thermistor. Readings are pushed over Wi-Fi to a Supabase backend, which evaluates them against per-patient thresholds and raises alerts. Clinicians view live vitals, trends, and alerts in a responsive Next.js web dashboard. The bedside unit also drives a local red/green LED and buzzer, so an out-of-range reading is still visible at the bed even if the network or dashboard is unavailable.

# 2. Technology stack

| Layer | Technology | Purpose |
| --- | --- | --- |
| Sensing | MAX30100 pulse oximeter (I2C) | Heart rate (bpm) and blood oxygen concentration / SpO2 (%) |
| Sensing | NTC thermistor + 4.7k divider (ADC) | Body temperature via Steinhart-Hart approximation |
| Microcontroller | ESP32-DevKitC32D | Reads both sensors, runs Wi-Fi, drives local alarm indicators |
| Firmware framework | Arduino core + PlatformIO | Build system and hardware abstraction for the ESP32 |
| Firmware libraries | Arduino-MAX30100, ArduinoJson, WiFi.h, HTTPClient.h | Sensor driver, JSON payload encoding, network transport |
| Local alerting | GPIO-driven LED pair + buzzer transistor drivers | Bedside red/green/buzzer feedback even if network is down |
| Transport | HTTPS POST to Supabase REST (PostgREST) | Sends each reading straight from the ESP32, no custom backend server |
| Backend / database | Supabase (PostgreSQL, Auth, Realtime, Row Level Security) | Stores patients, devices, readings, alerts; enforces access rules |
| Backend logic | Postgres triggers/functions (resolve_vitals_device, evaluate_vitals_reading) | Maps a device_key to a patient and raises threshold-breach alerts server-side |
| Web framework | Next.js 16 (App Router), React 19, TypeScript | Clinician-facing dashboard, server components + server actions |
| Styling / UI kit | Tailwind CSS v4, shadcn/ui primitives | Responsive, accessible component library used across the dashboard |
| Data viz | Recharts | Historical vitals trend charts on the patient detail page |
| Realtime | Supabase Realtime (Postgres logical replication) | Live vitals + alert updates in the browser without polling |
| Auth | Supabase Auth (email/password) + Next.js proxy.ts | Clinician login/session, protects the dashboard routes |

# 3. Data flow

- MAX30100 (I2C) and thermistor (ADC) are sampled every ~3 seconds by the ESP32.

- The ESP32 POSTs a JSON reading directly to Supabase's REST API (PostgREST), authenticated with the project's anon key plus a per-device secret (device_key).

- A Postgres trigger (resolve_vitals_device) looks up the device_key, attaches the correct patient_id and device_id to the row, and rejects unknown keys.

- A second trigger (evaluate_vitals_reading) compares the reading to that patient's thresholds (heart rate, SpO2, temperature) and inserts a warning/critical alert row when one is breached.

- Supabase Realtime streams new readings and alerts to the Next.js dashboard, where clinicians see live vitals cards, trend charts, and an alerts feed with toast notifications, and can acknowledge alerts.

- Row Level Security ensures the anon key (used by the ESP32) can only insert readings - it can never read patient data. Clinicians authenticate via Supabase Auth to read/write through the dashboard.

# 4. Firmware code

The firmware is a PlatformIO project targeting the ESP32-DevKitC32D (env:esp32dev, Arduino framework). Full source below.

## 4.1 platformio.ini

; Patient Monitor firmware - ESP32-DevKitC

; Build with PlatformIO: `pio run` / `pio run -t upload`

[env:esp32dev]

platform = espressif32

board = esp32dev

framework = arduino

monitor_speed = 115200

lib_deps =

bblanchon/ArduinoJson @ ^7.1.0

oxullo/Arduino-MAX30100 @ ^1.2.1

build_flags =

-D CORE_DEBUG_LEVEL=1

## 4.2 include/config.h

Wi-Fi credentials, Supabase URL/anon key, device key, pin map, thermistor constants, and clinical thresholds.

#pragma once

// ---------------------------------------------------------------------------

// Wi-Fi

// ---------------------------------------------------------------------------

#define WIFI_SSID     "YOUR_WIFI_SSID"

#define WIFI_PASSWORD "YOUR_WIFI_PASSWORD"

// ---------------------------------------------------------------------------

// Supabase

// ---------------------------------------------------------------------------

// Project URL and anon key, from Project Settings -> API.

// The anon key is safe to embed on-device: RLS only allows it to INSERT into

// vitals_readings, gated by DEVICE_KEY below (see supabase/migrations/0001_init.sql).

#define SUPABASE_URL      "https://yrcllbptaocqzjmdqgys.supabase.co"

#define SUPABASE_ANON_KEY "eyJhbGciOiJIUzI1NiIsInR5cCI6IkpXVCJ9.eyJpc3MiOiJzdXBhYmFzZSIsInJlZiI6InlyY2xsYnB0YW9jcXpqbWRxZ3lzIiwicm9sZSI6ImFub24iLCJpYXQiOjE3ODM5NjQwMjksImV4cCI6MjA5OTU0MDAyOX0.88-sRqLw4sX1shXWeCyMKq90BxWdNW7jRLXkvl-w5V8"

// Per-device secret. Create a row in the `devices` table from the web app

// (Devices page) and paste its generated device_key here. This is how the

// backend maps a reading to the correct patient without trusting the ESP32

// to say who it is.

#define DEVICE_KEY "PASTE_DEVICE_KEY_HERE"

// ---------------------------------------------------------------------------

// Pin map - matches the schematic in docs/. Adjust to your wiring.

// ---------------------------------------------------------------------------

#define PIN_I2C_SDA   21   // MAX30100 SDA (J1/J4)

#define PIN_I2C_SCL   22   // MAX30100 SCL (J1/J4)

#define PIN_TEMP_ADC  34   // NTC thermistor divider (J3/R7), ADC1-only pin

#define PIN_BUZZER    19   // Buzzer driver transistor Q3 (R9)

#define PIN_LED_RED   18   // Alert LED, driven by Q1 (R4/D4)

#define PIN_LED_GREEN 5    // Status-OK LED, driven by Q2 (R5/D3)

// ---------------------------------------------------------------------------

// Thermistor (NTC) - series resistor R7 = 4.7k to +5V, thermistor to GND

// ---------------------------------------------------------------------------

#define SERIES_RESISTOR_OHMS     4700.0

#define THERMISTOR_NOMINAL_OHMS  10000.0 // resistance at 25C, check your NTC datasheet

#define THERMISTOR_NOMINAL_TEMP  25.0

#define THERMISTOR_B_COEFFICIENT 3950.0  // typical for 10k NTC, check your datasheet

#define ADC_MAX_VALUE            4095.0

// ---------------------------------------------------------------------------

// Clinical thresholds used for on-device LED/buzzer alerting.

// The web app / database applies the authoritative per-patient thresholds;

// these are just sane local defaults so the unit still beeps if offline.

// ---------------------------------------------------------------------------

#define HR_MIN 50

#define HR_MAX 120

#define SPO2_MIN 92

#define TEMP_MIN_C 35.5

#define TEMP_MAX_C 38.5

// ---------------------------------------------------------------------------

// Timing

// ---------------------------------------------------------------------------

#define REPORT_INTERVAL_MS  3000   // how often a reading is sent to Supabase

#define WIFI_RETRY_MS       10000

## 4.3 src/main.cpp

Sensor sampling, Steinhart-Hart temperature conversion, local LED/buzzer alerting, and the Supabase POST.

// Patient Monitor - ESP32 firmware

//

// Reads pulse rate + SpO2 from a MAX30100 and body temperature from an NTC

// thermistor divider, pushes readings to Supabase, and drives a local

// red/green LED + buzzer so bedside staff get an alert even if the network

// or web app is down. See docs/hardware.md for the pin map.

#include <Arduino.h>

#include <WiFi.h>

#include <HTTPClient.h>

#include <Wire.h>

#include <ArduinoJson.h>

#include <MAX30100_PulseOximeter.h>

#include "config.h"

PulseOximeter pox;

uint32_t lastReportAt = 0;

uint32_t lastBeepToggleAt = 0;

bool buzzerOn = false;

void onBeatDetected() {

Serial.println("Beat detected");

}

void connectWiFi() {

if (WiFi.status() == WL_CONNECTED) return;

Serial.printf("Connecting to Wi-Fi \"%s\"...\n", WIFI_SSID);

WiFi.mode(WIFI_STA);

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

uint32_t start = millis();

while (WiFi.status() != WL_CONNECTED && millis() - start < WIFI_RETRY_MS) {

delay(250);

Serial.print(".");

}

Serial.println();

if (WiFi.status() == WL_CONNECTED) {

Serial.printf("Wi-Fi connected, IP=%s\n", WiFi.localIP().toString().c_str());

} else {

Serial.println("Wi-Fi connect failed, will retry");

}

}

// Steinhart-Hart approximation for a series-resistor NTC divider.

float readTemperatureC() {

int raw = analogRead(PIN_TEMP_ADC);

if (raw <= 0 || raw >= (int)ADC_MAX_VALUE) {

return NAN; // open circuit / short - sensor fault

}

float resistance = SERIES_RESISTOR_OHMS * (ADC_MAX_VALUE / (float)raw - 1.0);

float steinhart = resistance / THERMISTOR_NOMINAL_OHMS;

steinhart = log(steinhart);

steinhart /= THERMISTOR_B_COEFFICIENT;

steinhart += 1.0 / (THERMISTOR_NOMINAL_TEMP + 273.15);

steinhart = 1.0 / steinhart;

steinhart -= 273.15;

return steinhart;

}

bool isOutOfRange(int hr, float spo2, float tempC, bool hasContact) {

if (!hasContact) return false;

if (hr > 0 && (hr < HR_MIN || hr > HR_MAX)) return true;

if (spo2 > 0 && spo2 < SPO2_MIN) return true;

if (!isnan(tempC) && (tempC < TEMP_MIN_C || tempC > TEMP_MAX_C)) return true;

return false;

}

void updateIndicators(bool alarm) {

digitalWrite(PIN_LED_GREEN, alarm ? LOW : HIGH);

digitalWrite(PIN_LED_RED, alarm ? HIGH : LOW);

if (!alarm) {

digitalWrite(PIN_BUZZER, LOW);

buzzerOn = false;

return;

}

// Pulse the buzzer instead of a continuous tone.

if (millis() - lastBeepToggleAt > 400) {

buzzerOn = !buzzerOn;

digitalWrite(PIN_BUZZER, buzzerOn ? HIGH : LOW);

lastBeepToggleAt = millis();

}

}

void postReading(int hr, float spo2, float tempC, const char *signalQuality) {

if (WiFi.status() != WL_CONNECTED) return;

HTTPClient http;

String url = String(SUPABASE_URL) + "/rest/v1/vitals_readings";

http.begin(url);

http.addHeader("Content-Type", "application/json");

http.addHeader("apikey", SUPABASE_ANON_KEY);

http.addHeader("Authorization", String("Bearer ") + SUPABASE_ANON_KEY);

http.addHeader("Prefer", "return=minimal");

JsonDocument doc;

doc["device_key"] = DEVICE_KEY;

if (hr > 0) doc["heart_rate"] = hr;

if (spo2 > 0) doc["spo2"] = spo2;

if (!isnan(tempC)) doc["temperature"] = round(tempC * 10) / 10.0;

doc["signal_quality"] = signalQuality;

String body;

serializeJson(doc, body);

int code = http.POST(body);

if (code < 200 || code >= 300) {

Serial.printf("Supabase insert failed, HTTP %d: %s\n", code, http.getString().c_str());

}

http.end();

}

void setup() {

Serial.begin(115200);

delay(200);

pinMode(PIN_LED_RED, OUTPUT);

pinMode(PIN_LED_GREEN, OUTPUT);

pinMode(PIN_BUZZER, OUTPUT);

digitalWrite(PIN_LED_RED, LOW);

digitalWrite(PIN_LED_GREEN, LOW);

digitalWrite(PIN_BUZZER, LOW);

analogReadResolution(12);

connectWiFi();

Wire.begin(PIN_I2C_SDA, PIN_I2C_SCL);

if (!pox.begin()) {

Serial.println("MAX30100 init failed, check wiring");

} else {

pox.setIRLedCurrent(MAX30100_LED_CURR_7_6MA);

pox.setOnBeatDetectedCallback(onBeatDetected);

Serial.println("MAX30100 ready");

}

}

void loop() {

connectWiFi();

pox.update();

if (millis() - lastReportAt < REPORT_INTERVAL_MS) {

// Still keep indicators live between reports using latest cached values.

return;

}

lastReportAt = millis();

int hr = (int)round(pox.getHeartRate());

float spo2 = pox.getSpO2();

float tempC = readTemperatureC();

bool hasContact = hr > 0 && spo2 > 0;

const char *signalQuality = hasContact ? "ok" : "no_contact";

bool alarm = isOutOfRange(hr, spo2, tempC, hasContact);

updateIndicators(alarm);

Serial.printf("HR=%d SpO2=%.1f Temp=%.1fC quality=%s alarm=%d\n",

hr, spo2, tempC, signalQuality, alarm);

postReading(hr, spo2, tempC, signalQuality);

}